Wi-Fi Concepts

Questions & Answers

Q1. In which OSI layer does the Wi-Fi standard/protocol fit?

Wi-Fi (IEEE 802.11) primarily operates at Layer 1 (Physical) and Layer 2 (Data Link) of the OSI model.

- Physical Layer (Layer 1): Handles the actual radio signal transmission, frequency bands, modulation techniques (OFDM, DSSS), and encoding of data into wireless signals.

- Data Link Layer (Layer 2): Specifically the MAC (Medium Access Control) sublayer — manages frame addressing, collision avoidance using CSMA/CA, and access control to the wireless medium.

Q2. Wi-Fi Devices and Matching to Wi-Fi Generations

Here's a general mapping based on common devices most people use:

| Device | Standard | Wi-Fi Generation | Band | Max Speed (approx) |
| --- | --- | --- | --- | --- |
| Smartphone (mid-range ~2021) | 802.11ac | Wi-Fi 5 | 2.4 / 5 GHz | ~433 Mbps – 1.3 Gbps |
| Laptop (recent ~2022+) | 802.11ax | Wi-Fi 6 | 2.4 / 5 / 6 GHz | Up to 9.6 Gbps |
| Smart TV | 802.11ac | Wi-Fi 5 | 2.4 / 5 GHz | ~600 Mbps |
| Home Router | 802.11ax | Wi-Fi 6 | Dual/Tri band | Up to 9.6 Gbps |
| Older budget phone | 802.11n | Wi-Fi 4 | 2.4 GHz | ~150 Mbps |

How to check on  device:

- Windows: Settings → Network → Wi-Fi → Hardware Properties. You'll see 'Protocol' listed as 802.11ax/ac/n etc.

- Android: Settings → About Phone → Wi-Fi specs, or use an app like 'Wi-Fi Analyzer'.

Q3. What is BSS and ESS?

BSS – Basic Service Set

It's one access point (AP) with all the devices connected to it. Every BSS has a unique identifier called a BSSID, which is essentially the MAC address of the AP.

There are two types:

- IBSS (Independent BSS): Devices talk directly to each other with no AP involved — like a mobile hotspot between two laptops.

- Infrastructure BSS: Devices connect through a central access point (the normal home Wi-Fi setup).

ESS – Extended Service Set

When you have multiple APs covering a larger area (like a college campus or a big office) all sharing the same SSID (network name), that's an ESS. The APs are connected via a wired backbone called the Distribution System. This is how seamless roaming works — your phone stays on the same network as you walk from one room to another.

Q4. What are the basic functionalities of a Wi-Fi Access Point?

A Wi-Fi Access Point (AP) does quite a few things:

- Wireless to Wired Bridging: Connects Wi-Fi clients to the wired LAN/internet backbone.

- SSID Broadcasting: Advertises the network name so nearby devices can discover and connect to it.

- Authentication & Association: Verifies clients using passwords (WPA2/WPA3) and assigns them to the network.

- Traffic Management: Forwards data frames between connected clients and the router/internet.

- DHCP Relay / IP Assignment: In many home setups, the AP/router assigns IP addresses to devices.

- Band Steering: Automatically pushes capable devices to the faster 5 GHz band.

- QoS (Quality of Service): Prioritizes traffic like video calls over file downloads.

- Security: Implements encryption (WPA3), MAC filtering, and guest network isolation.

Q5. Difference between Bridge Mode and Repeater Mode

| Feature | Bridge Mode | Repeater Mode |
| --- | --- | --- |
| Purpose | Connects two separate networks | Extends range of an existing network |
| How it works | Links two LANs (wired to wireless or vice versa) | Receives Wi-Fi signal and rebroadcasts it |
| NAT/DHCP | Disabled — passes traffic through | Usually disabled to avoid double NAT |
| Speed impact | Minimal | ~50% bandwidth loss on extended segment |
| Use case | Connecting a wired device block to wireless | Eliminating dead zones in a large home |
| Same SSID? | Not necessarily | Yes, same network name |

Q6. Differences between 802.11a and 802.11b

| Feature | 802.11a | 802.11b |
| --- | --- | --- |
| Year released | 1999 | 1999 |
| Frequency Band | 5 GHz | 2.4 GHz |
| Max Speed | 54 Mbps | 11 Mbps |
| Range | Shorter (high freq = less wall penetration) | Longer (low freq = better coverage) |
| Interference | Less (5 GHz less crowded) | More (shares band with microwaves, Bluetooth) |
| Modulation | OFDM | DSSS |
| Compatibility | Not compatible with 802.11b | Not compatible with 802.11a |
| Adoption | More expensive, less popular | Cheaper, widely adopted |

Q7. 2.4 GHz vs 5 GHz — Observed Differences

Here's what you'd typically observe when connecting to the same router on both bands:

| Property | 2.4 GHz | 5 GHz |
| --- | --- | --- |
| Frequency | 2.4 GHz | 5 GHz |
| Channel Width | 20 / 40 MHz | 20 / 40 / 80 / 160 MHz |
| Max Negotiated Speed | ~150–300 Mbps (Wi-Fi 4) | ~433 Mbps–1.3 Gbps (Wi-Fi 5/6) |
| Signal Strength (RSSI) | Higher (e.g., -45 dBm) | Slightly lower (e.g., -55 dBm) |
| Range | Better / Longer | Shorter |
| Interference | Higher | Lower |
| Latency (ping) | Slightly higher | Slightly lower |
| Standard typically shown | 802.11n (Wi-Fi 4) | 802.11ac/ax (Wi-Fi 5/6) |
| Common channels used | 1, 6, or 11 | 36, 40, 44, 149, etc. |

How to check on Windows: After connecting, go to Settings → Network → Wi-Fi → Hardware Properties. It shows Protocol, Band, Channel, and Link Speed.

Q8. Difference between IEEE and WFA

|  | IEEE | WFA (Wi-Fi Alliance) |
| --- | --- | --- |
| Full Form | Institute of Electrical and Electronics Engineers | Wi-Fi Alliance |
| Type | Standards body / technical organization | Industry consortium / trade group |
| Role | Defines the technical standards (802.11a/b/g/n/ac/ax) | Certifies products implement those standards correctly |
| Output | 802.11 technical specifications | Wi-Fi CERTIFIED™ logo and branding |
| Who they are | Engineers, researchers, academics | Device manufacturers (Intel, Qualcomm, Apple, etc.) |
| Example | Defined 802.11ax (the technical spec) | Branded it as 'Wi-Fi 6' and certifies devices |

Q9. Types of Wi-Fi Internet Connectivity Backhaul

"Backhaul" refers to how the internet signal gets from the ISP to your access point(s).

| Backhaul Type | Description | Example |
| --- | --- | --- |
| Fiber (FTTH/FTTP) | Optical fiber directly to home — very fast and reliable | Jio Fiber, ACT Fibernet |
| Cable (DOCSIS) | Coaxial cable from ISP to home | Hathway, BSNL cable broadband |
| DSL (ADSL/VDSL) | Over existing telephone copper lines | BSNL Broadband (older areas) |
| Fixed Wireless | Microwave/radio link from ISP tower to home antenna | Local ISPs in tier-2/3 cities |
| 4G/5G LTE | SIM-based router using cellular mobile data | JioFi, Airtel 4G hotspot |
| Satellite | Internet via satellite — high latency | Starlink, BSNL VSAT |

For a typical home/college in Hyderabad:

- Home connections are commonly Fiber (ACT Fibernet / Jio Fiber) or 4G LTE (JioFi).

- Colleges typically use Fiber with enterprise switches as the backbone, then APs distribute Wi-Fi campus-wide.

- You can verify your backhaul by checking the WAN port type on your router or asking your ISP.

Q10. Wi-Fi Topologies and Use Cases

| Topology | Description | Use Case |
| --- | --- | --- |
| IBSS (Ad-Hoc) | Devices connect directly to each other — no AP needed | File sharing between two laptops, temporary offline networks |
| BSS (Infrastructure) | All devices connect through a single access point | Home Wi-Fi, small office setup |
| ESS (Extended) | Multiple APs on same SSID via wired backbone | College campus, hotels, large office buildings |
| Mesh Network | APs communicate wirelessly with each other — no cables needed | Large homes, smart home setups (Google Nest, Eero) |
| WDS (Wireless Distribution System) | APs linked wirelessly to extend coverage | Buildings where running cables is difficult or costly |
| Point-to-Point (PtP) | Two directional antennas connecting two locations wirelessly | Connecting two buildings across a campus or road |
| Point-to-Multipoint (PtMP) | One central AP communicates with multiple remote nodes | Rural internet distribution, ISP tower broadcasting |