802.11 PHY Layer

Standards, Modulation & Frame Structure — Questions & Answers

Q1. What are the Different 802.11 PHY Layer Standards? Compare Their Characteristics.

The 802.11 PHY (Physical) layer is what actually puts the signal on air. Every generation of Wi-Fi has upgraded the PHY layer to squeeze more speed, better range, or more efficiency out of the radio. Here's how they stack up:

| Standard | Wi-Fi Gen | Year | Band | Max Speed | Modulation | Key Feature |
| --- | --- | --- | --- | --- | --- | --- |
| 802.11 (legacy) | — | 1997 | 2.4 GHz | 2 Mbps | FHSS / DSSS | The very first Wi-Fi spec |
| 802.11a | — | 1999 | 5 GHz | 54 Mbps | OFDM | First to use 5 GHz and OFDM |
| 802.11b | — | 1999 | 2.4 GHz | 11 Mbps | DSSS (CCK) | Became the dominant standard; cheap hardware |
| 802.11g | — | 2003 | 2.4 GHz | 54 Mbps | OFDM | Backward compatible with 802.11b |
| 802.11n | Wi-Fi 4 | 2009 | 2.4 / 5 GHz | 600 Mbps | OFDM + MIMO | First to introduce MIMO (up to 4 streams) |
| 802.11ac | Wi-Fi 5 | 2013 | 5 GHz only | 6.9 Gbps | OFDM + MU-MIMO | Wider channels (up to 160 MHz), 256-QAM |
| 802.11ax | Wi-Fi 6/6E | 2019/2021 | 2.4/5/6 GHz | 9.6 Gbps | OFDMA + MU-MIMO | OFDMA, BSS Coloring, TWT for IoT devices |
| 802.11be | Wi-Fi 7 | 2024 | 2.4/5/6 GHz | 46 Gbps | OFDMA + 4096-QAM | Multi-Link Operation (MLO), 320 MHz channels |

Q2. What are DSSS and FHSS? How Do They Work?

Both DSSS and FHSS are spread spectrum techniques — meaning they deliberately spread a signal over a wider frequency range than strictly necessary. The goal is resistance to interference and eavesdropping. They were the two main approaches used in early 802.11 Wi-Fi.

FHSS — Frequency Hopping Spread Spectrum

The idea here is simple: don't stay on one frequency. The transmitter and receiver agree on a hopping pattern (a sequence of frequencies to jump through), and they hop together in sync — typically hundreds of times per second.

- How it works:: The 2.4 GHz band is divided into 79 channels, each 1 MHz wide. The transmitter sends a little data on channel 5, then hops to channel 38, then to channel 12 — following the agreed sequence.

- Why it helps:: An interferer on a fixed frequency will only disrupt the signal for the brief moment you're on that frequency. The rest of the hops are unaffected.

- Downside:: Speed is limited — FHSS topped out at 2 Mbps and was phased out after 802.11's first version.

DSSS — Direct Sequence Spread Spectrum

DSSS takes a different approach: instead of hopping frequencies, it spreads the signal across a wider bandwidth by mixing it with a high-rate pseudorandom code called a chipping sequence.

- How it works:: Every bit of your data is multiplied by an 11-chip Barker code. So one bit becomes 11 chips transmitted at a much faster rate across a wider slice of spectrum (~22 MHz).

- Why it helps:: If some chips are corrupted by interference, the receiver can still reconstruct the original bit using the known chipping code. It's inherently noise-resistant.

- Used in:: 802.11b (with CCK encoding on top) — this is what made 802.11b so resilient and popular in real-world environments.

- Downside:: Less spectrally efficient than OFDM; replaced by OFDM in 802.11a/g and beyond.

| Feature | FHSS | DSSS |
| --- | --- | --- |
| Core idea | Rapidly hops between frequencies | Spreads signal using a chipping code over wide bandwidth |
| Bandwidth used | 1 MHz per hop | ~22 MHz fixed |
| Interference resistance | Good — interference only hits during that hop | Good — corrupted chips are correctable |
| Max speed | 2 Mbps | 11 Mbps (with CCK in 802.11b) |
| Used in | Original 802.11 (1997) | 802.11b (1999) |
| Status | Obsolete | Obsolete (replaced by OFDM) |

Q3. How Do Modulation Schemes Work in the PHY Layer? Compare Different Schemes Across Wi-Fi Standards.

Modulation is how digital data (0s and 1s) gets encoded onto a radio wave. The radio wave has three properties you can manipulate: amplitude (height), frequency, and phase. Different modulation schemes use one or more of these to carry data.

The main modulation schemes used in Wi-Fi:

- BPSK (Binary Phase Shift Keying):: Changes the phase of the wave to represent 0 or 1. Only 2 possible states → 1 bit per symbol. Very robust, used in low-SNR conditions.

- QPSK (Quadrature PSK):: Uses 4 phase positions → 2 bits per symbol. Still quite robust, used for control frames.

- 16-QAM:: Combines amplitude and phase changes → 4 bits per symbol. Faster, needs a better signal.

- 64-QAM:: 64 possible amplitude+phase combinations → 6 bits per symbol. Used in 802.11n/ac at good signal strength.

- 256-QAM:: 256 combinations → 8 bits per symbol. Introduced in 802.11ac Wave 2. Needs very clean signal (high SNR).

- 1024-QAM:: 10 bits per symbol. Introduced in Wi-Fi 6 (802.11ax). Requires excellent SNR.

- 4096-QAM:: 12 bits per symbol. Wi-Fi 7 (802.11be). Only practical at very close range with near-perfect conditions.

The general rule: higher-order QAM = more bits per symbol = faster speeds, but needs a stronger, cleaner signal. Your device automatically picks the highest modulation it can sustain given the current signal quality.

| Modulation | Bits/Symbol | Wi-Fi Standard | Required SNR (approx) | Use case |
| --- | --- | --- | --- | --- |
| BPSK | 1 | 802.11a/b/g/n/ac/ax | ~5 dB | Long range, weak signal, control frames |
| QPSK | 2 | 802.11a/b/g/n/ac/ax | ~10 dB | Medium range, moderate signal |
| 16-QAM | 4 | 802.11a/g/n/ac/ax | ~17 dB | Good signal, moderate throughput |
| 64-QAM | 6 | 802.11a/g/n/ac/ax | ~25 dB | Strong signal |
| 256-QAM | 8 | 802.11ac/ax | ~30 dB | Very strong signal, high throughput |
| 1024-QAM | 10 | 802.11ax (Wi-Fi 6) | ~35 dB | Excellent signal, dense environments |
| 4096-QAM | 12 | 802.11be (Wi-Fi 7) | ~40 dB | Near-perfect conditions, ultra-high throughput |

Q4. What is the Significance of OFDM in WLAN? How Does it Improve Performance?

OFDM — Orthogonal Frequency Division Multiplexing — is arguably the most important technology in modern Wi-Fi. It replaced DSSS starting with 802.11a and has been the backbone of every standard since (802.11a/g/n/ac/ax all use it in some form).

What OFDM actually does:

Instead of transmitting data on a single wide carrier (which is vulnerable to fading and interference at specific frequencies), OFDM splits the channel into many narrow parallel subcarriers — each carrying a small piece of the data simultaneously.

- Example:: A 20 MHz 802.11n channel is divided into 64 subcarriers, each just 312.5 kHz wide. Data is spread across all of them in parallel.

- Orthogonal:: The subcarriers are mathematically orthogonal — their peaks align with the nulls of neighboring subcarriers. This means they don't interfere with each other despite being very closely spaced, saving spectrum.

Why this matters:

- Multipath resistance:: In buildings, signals bounce off walls and arrive at the receiver at slightly different times (multipath). A single-carrier system gets confused by this. OFDM's narrow subcarriers have a long symbol duration, so small time delays don't corrupt the data.

- Frequency selective fading:: If interference kills a few subcarriers, only a small portion of the data is affected — not the entire transmission. Error correction codes recover the lost bits.

- Spectral efficiency:: Because subcarriers are orthogonal and tightly packed, OFDM uses spectrum very efficiently compared to traditional FDM.

- Scalability:: The same OFDM framework scales beautifully — wider channels (40, 80, 160 MHz) just mean more subcarriers, not a different technology.

OFDM is why Wi-Fi works well inside buildings full of walls, furniture, and reflective surfaces. It was designed specifically for these kinds of environments.

Q5. How are Frequency Bands Divided for Wi-Fi? Explain Different Bands and Their Channels.

Wi-Fi operates across three frequency bands: 2.4 GHz, 5 GHz, and 6 GHz (the newest). Each has a different balance of range, speed, and congestion.

2.4 GHz Band

- Range:: 83.5 MHz total (2.401–2.4835 GHz), divided into 14 channels of 22 MHz width each (varies by country).

- Non-overlapping channels:: Only channels 1, 6, and 11 don't overlap in most of the world. Every other channel causes inter-channel interference with its neighbors.

- Pros:: Better range and wall penetration. Works with older devices.

- Cons:: Only 3 usable non-overlapping channels. Heavily congested — shared with Bluetooth, microwaves, baby monitors, and every neighbour's router.

5 GHz Band

- Range:: Much wider — roughly 500 MHz of usable spectrum split into channels numbered 36 to 177.

- Non-overlapping channels:: 25 non-overlapping 20 MHz channels available (varies by country and regulatory domain). With 80 MHz bonding, you get about 6 non-overlapping channels.

- Pros:: Far less congested. Supports wider channels (80/160 MHz) for much higher throughput.

- Cons:: Shorter range (higher frequency = more attenuation through walls). Some channels (DFS channels) require radar detection.

6 GHz Band (Wi-Fi 6E and Wi-Fi 7)

- Range:: Massive — 1200 MHz of new spectrum (5.925–7.125 GHz), allowing up to 59 non-overlapping 20 MHz channels.

- Non-overlapping channels:: 7 non-overlapping 160 MHz channels. This is a game changer for high-density deployments.

- Pros:: Virtually no congestion (legacy devices can't use it). Huge bandwidth available. Wi-Fi 7 uses 320 MHz channels here.

- Cons:: Shortest range of the three. Walls affect it more. Newer devices only.

| Band | Frequency Range | Total Spectrum | Non-overlapping 20MHz Ch. | Max Channel Width | Wi-Fi Standards |
| --- | --- | --- | --- | --- | --- |
| 2.4 GHz | 2.401–2.483 GHz | ~83 MHz | 3 (ch 1, 6, 11) | 40 MHz | 802.11b/g/n/ax |
| 5 GHz | 5.150–5.850 GHz | ~500 MHz | Up to 25 | 160 MHz | 802.11a/n/ac/ax |
| 6 GHz | 5.925–7.125 GHz | ~1200 MHz | Up to 59 | 320 MHz (Wi-Fi 7) | 802.11ax (6E), 802.11be |

Q6. What is the Role of Guard Intervals in WLAN Transmission? How Does a Short Guard Interval Improve Efficiency?

When OFDM transmits data, it does so in discrete chunks called symbols. After each symbol is sent, there's a deliberate pause before the next one — this pause is the Guard Interval (GI).

In a real-world environment, the radio signal bounces off walls, furniture, and ceilings before reaching the receiver — a phenomenon called multipath. These reflections arrive slightly after the direct signal. If the next symbol starts before all the reflections from the previous symbol have died out, the reflections from symbol N bleed into symbol N+1, causing Inter-Symbol Interference (ISI) — which corrupts the data.

The guard interval is basically a silent buffer between symbols to let those reflections die down before the next symbol starts.

Standard GI vs Short GI:

| Guard Interval | Duration | Used In | When to use it | Throughput gain |
| --- | --- | --- | --- | --- |
| Standard GI (Long) | 800 ns | All 802.11 standards | Environments with high multipath (large rooms, warehouses) | Baseline |
| Short GI (SGI) | 400 ns | 802.11n, 802.11ac, 802.11ax | Clean environments, short range, low multipath | ~11% throughput improvement |

How Short GI improves efficiency:

By cutting the guard interval from 800 ns to 400 ns, you're spending less time doing nothing between symbols. The radio can start transmitting the next symbol sooner, fitting more symbols into the same time period — directly translating to higher data throughput.

- Example:: In 802.11n with a 20 MHz channel, standard GI gives a max of 65 Mbps per spatial stream. With short GI, that increases to 72.2 Mbps — the same 11% improvement.

- Risk:: If you use short GI in a high-multipath environment, reflections from the previous symbol will bleed into the next, causing errors. Your device negotiates GI automatically based on conditions.

Q7. Describe the Structure of an 802.11 PHY Layer Frame. What are its Key Components?

When data moves through the PHY layer, it's packaged into a specific frame format called a PPDU (PHY Protocol Data Unit). The exact structure varies between Wi-Fi generations, but every PPDU has the same three main sections:

| Section | Full Name | What it contains | Purpose |
| --- | --- | --- | --- |
| PLCP Preamble | Physical Layer Convergence Procedure Preamble | Training sequences (short + long training fields) | Helps the receiver synchronize, detect the start of the frame, and estimate the channel |
| PLCP Header / SIG Field | PLCP Header or Signal Field | Modulation, coding rate, data length, HT/VHT/HE format indicators | Tells the receiver how to decode the rest of the frame |
| PSDU / Data Field | PHY Service Data Unit | The actual MAC frame payload (encrypted data) | The payload being transmitted — this is what the user sent |

Key components explained:

- Short Training Field (STF):: A known pattern repeated 10 times, used for signal detection, AGC (automatic gain control), and coarse frequency sync.

- Long Training Field (LTF):: Used for precise channel estimation — the receiver uses this to understand how the channel is distorting the signal and compensates accordingly.

- SIG / Signal Field:: Contains metadata about the frame — what MCS (modulation and coding scheme) was used, how long the frame is, whether it uses STBC or LDPC, etc.

- Data / PSDU:: The actual payload. This is where your YouTube video, webpage, or chat message lives.

With each new Wi-Fi generation (HT for 802.11n, VHT for 802.11ac, HE for 802.11ax), more SIG fields and LTF sequences were added to support new features like MIMO streams and OFDMA resource units.

Q8. What is the Difference Between OFDM and OFDMA?

This is one of the most important distinctions between Wi-Fi 5 (802.11ac) and Wi-Fi 6 (802.11ax). Both use OFDM as the underlying technology, but OFDMA changes how that spectrum is shared between users.

OFDM — One user at a time per channel:

In standard OFDM (used in 802.11a/g/n/ac), the entire channel — all its subcarriers — is assigned to one device at a time. If you have 20 devices trying to talk, they take turns. Device A gets the whole 80 MHz channel for its transmission, then device B does, and so on. This is fine when devices have large amounts of data to send, but it's wasteful for small-packet traffic (like IoT sensors or frequent acknowledgements).

OFDMA — Multiple users simultaneously on the same channel:

OFDMA (introduced in 802.11ax / Wi-Fi 6) divides the channel subcarriers into groups called Resource Units (RUs). Different RUs can be assigned to different devices at the same time. The AP acts as a scheduler — it allocates slices of the channel to multiple clients simultaneously.

| Feature | OFDM | OFDMA |
| --- | --- | --- |
| Channel allocation | Entire channel to one device at a time | Channel split into RUs, multiple devices served simultaneously |
| Efficiency for small packets | Poor — full channel used even for tiny packets | Great — small RU assigned to small-packet devices |
| Latency | Higher in dense networks (devices queue up) | Lower — more devices served per transmission window |
| Best for | Single user, large file transfers | Dense environments: offices, stadiums, classrooms with many devices |
| Used in | 802.11a/g/n/ac (Wi-Fi 4/5 and earlier) | 802.11ax (Wi-Fi 6/6E) and 802.11be (Wi-Fi 7) |
| Scheduling required? | No — first-come-first-served | Yes — AP must schedule which RUs go to which clients |

Analogy: OFDM is like a single-lane road where only one car moves at a time. OFDMA is like a multi-lane highway where many cars move simultaneously — some in big lanes, some in small ones, depending on how much they need.

Q9. What is the Difference Between MIMO and MU-MIMO?

MIMO — Multiple Input Multiple Output — is the technology of using multiple antennas to send and receive multiple data streams simultaneously. It's been in Wi-Fi since 802.11n (Wi-Fi 4). MU-MIMO is an evolution that extends this capability to multiple users at once.

MIMO (SU-MIMO — Single User):

- All the spatial streams from the AP go to a single device at a time.

- An AP with 4x4 MIMO can send 4 parallel streams — but only to one client per transmission window.

- If you have 10 devices, they still take turns. MIMO just makes each turn faster.

MU-MIMO (Multi-User MIMO):

- The AP can split its spatial streams across multiple clients simultaneously, each receiving their own dedicated stream.

- A 4x4 MU-MIMO AP can serve 4 clients each with 1 stream, or 2 clients each with 2 streams — all at the same time.

- This dramatically improves efficiency in dense environments where many devices are competing.

| Feature | SU-MIMO (MIMO) | MU-MIMO |
| --- | --- | --- |
| Stands for | Single-User MIMO | Multi-User MIMO |
| Streams served to | One device at a time | Multiple devices simultaneously |
| Introduced in | 802.11n (Wi-Fi 4) | Downlink: 802.11ac (Wi-Fi 5) / Uplink: 802.11ax (Wi-Fi 6) |
| Max spatial streams | 4 (802.11n), 8 (802.11ac/ax) | 4 users DL (Wi-Fi 5), 8 users DL+UL (Wi-Fi 6) |
| Best for | Environments with a few high-demand devices | Dense environments — offices, classrooms, stadiums |
| Requirement | Just multiple antennas | Requires beamforming + channel feedback from clients |
| Uplink MU-MIMO? | No | Yes — added in Wi-Fi 6 (802.11ax) |

Key point: Wi-Fi 5 added downlink MU-MIMO (AP → devices). Wi-Fi 6 added uplink MU-MIMO (devices → AP). Both directions being multi-user is a big deal for crowded networks.

Q10. What are PPDU, PLCP, and PMD in the PHY Layer?

These three terms describe layers within the 802.11 PHY layer itself. The PHY layer isn't just one flat layer — it has internal sublayers, each with a specific job:

| Term | Full Name | Role | Analogy |
| --- | --- | --- | --- |
| PMD | Physical Medium Dependent sublayer | Deals directly with the radio medium — modulation, encoding, transmitting and receiving the actual RF signal. It's hardware-level. | The actual engine and wheels of a car — makes it physically move. |
| PLCP | Physical Layer Convergence Procedure sublayer | Acts as the interface between the MAC layer and the PMD. It packages MAC frames into PPDUs by adding preambles and headers, and handles synchronization. | The transmission system — translates driver input into wheel movement. |
| PPDU | PHY Protocol Data Unit | The actual complete frame that gets transmitted over the air. It's the output of the PLCP — contains the preamble, signal fields, and the data payload (PSDU). | The car itself, fully assembled and ready to drive. |

How they relate to each other:

- The MAC layer hands a MSDU (MAC Service Data Unit) down to the PLCP.

- The PLCP wraps it with a preamble and header to form a PPDU.

- The PPDU is passed to the PMD, which modulates it onto an RF carrier and transmits it.

- On the receiving end, the PMD demodulates the signal, the PLCP strips the header, and the MAC layer receives its data back.

Q11. What are the Types of PPDU? Explain the PPDU Frame Format Across Different Wi-Fi Generations.

The PPDU format has evolved with every major Wi-Fi generation. Each new format added new fields to support new capabilities like MIMO, wider channels, and multi-user transmission. The receiver figures out which format it's dealing with by looking at the preamble.

| PPDU Type | Standard | Wi-Fi Gen | Key Fields | New Capability Added |
| --- | --- | --- | --- | --- |
| Legacy PPDU (L-PPDU) | 802.11a/g | — | Short Training Field (STF), Long Training Field (LTF), Signal (SIG), Data | Baseline OFDM frame format |
| HT-PPDU | 802.11n | Wi-Fi 4 | L-STF, L-LTF, L-SIG, HT-SIG, HT-STF, HT-LTF × N, Data | MIMO support — multiple HT-LTFs for each spatial stream |
| VHT-PPDU | 802.11ac | Wi-Fi 5 | L-STF, L-LTF, L-SIG, VHT-SIG-A, VHT-STF, VHT-LTF × N, VHT-SIG-B, Data | Wider channels (80/160 MHz), 256-QAM, MU-MIMO DL |
| HE-PPDU | 802.11ax | Wi-Fi 6/6E | L-STF, L-LTF, L-SIG, RL-SIG, HE-SIG-A, HE-SIG-B (MU), HE-STF, HE-LTF × N, Data | OFDMA resource units, TWT, BSS Coloring, uplink MU-MIMO |
| EHT-PPDU | 802.11be | Wi-Fi 7 | Similar to HE + EHT-SIG, EHT-LTF, MLO fields | Multi-Link Operation, 320 MHz channels, 4096-QAM |

Why do legacy fields (L-STF, L-LTF, L-SIG) always appear first?

Every PPDU — even Wi-Fi 6 HE-PPDUs — starts with the old 802.11a legacy preamble fields. This is intentional: older devices that don't understand 802.11n/ac/ax can still read the L-SIG and know to stay quiet for the duration of the frame, preventing collisions. This backward compatibility mechanism is built into every generation.

Q12. How is the Data Rate Calculated?

The data rate in 802.11 Wi-Fi isn't a fixed number — it's the product of several physical layer parameters all multiplied together. Understanding this formula helps explain why your connection speed changes depending on conditions.

The Data Rate Formula:

Data Rate = (Number of Subcarriers) × (Bits per Subcarrier) × (Coding Rate) × (Number of Spatial Streams) × (1 / Symbol Duration)

Breaking down each factor:

- Number of subcarriers:: Depends on channel width. A 20 MHz OFDM channel has 52 data subcarriers. An 80 MHz channel has 234. Wider channel = more subcarriers = higher rate.

- Bits per subcarrier (Modulation):: Determined by the modulation scheme. BPSK = 1 bit, QPSK = 2 bits, 64-QAM = 6 bits, 256-QAM = 8 bits, 1024-QAM = 10 bits.

- Coding Rate:: Forward error correction adds redundancy bits. A coding rate of 3/4 means 3 out of every 4 bits are actual data (the rest are error correction). Higher coding rate = more data bits per symbol.

- Spatial Streams (MIMO):: Each additional antenna stream multiplies the data rate. 2 streams = 2× the rate of 1 stream (roughly).

- Symbol Duration:: Each OFDM symbol takes a fixed time to transmit. In 802.11n, one symbol = 3.2 µs + guard interval (0.8 or 0.4 µs). Shorter symbol time = more symbols per second = higher rate.

Worked Example — 802.11n, 20 MHz, 1 stream, 64-QAM, 3/4 coding, standard GI:

- 52 data subcarriers × 6 bits (64-QAM) × 3/4 coding rate = 234 bits per symbol

- Symbol duration = 3.2 µs + 0.8 µs GI = 4.0 µs → 250,000 symbols per second

- 234 bits × 250,000 = 58.5 Mbps

- With short GI (3.6 µs): 234 × 277,778 = 65 Mbps — which matches the known 802.11n MCS7 rate exactly.

| Parameter | Effect on Data Rate | Example Values |
| --- | --- | --- |
| Channel Width | More subcarriers → higher rate | 20, 40, 80, 160, 320 MHz |
| Modulation (MCS) | More bits per symbol → higher rate | BPSK (1 bit) to 4096-QAM (12 bits) |
| Coding Rate | Less overhead → more useful data | 1/2, 2/3, 3/4, 5/6 |
| Spatial Streams | Linear multiplier on rate | 1 to 8 streams (Wi-Fi 6) |
| Guard Interval | Shorter GI → more symbols/sec | 800 ns (LGI) vs 400 ns (SGI) vs 800/1600/3200 ns (HE) |

This is why Wi-Fi speed claims like '9.6 Gbps for Wi-Fi 6' are theoretical maximums — they assume 8 spatial streams, 1024-QAM, 160 MHz channel width, 5/6 coding rate, and short guard interval, all simultaneously. Real-world speeds are a fraction of that depending on your environment and device capability.