Wireless LAN Architecture

CAPWAP, WLC & AP Modes — Questions & Answers

Q1. Brief about SplitMAC Architecture and How it Improves AP Performance

In a traditional Wi-Fi setup, each access point (AP) handles everything on its own — authentication, encryption, roaming decisions, you name it. That works fine when you have 2–3 APs, but it becomes a management nightmare at scale.

SplitMAC architecture solves this by splitting the 802.11 MAC layer functions between the AP and a central Wireless LAN Controller (WLC):

| Function | Handled By | Examples |
| --- | --- | --- |
| Real-time / time-sensitive tasks | AP (locally) | Beacon transmission, ACK handling, probe responses, encryption/decryption, CSMA/CA |
| Management / intelligence tasks | WLC (centrally) | Authentication, association, roaming decisions, QoS policy, RF management, firmware updates |

How does this improve performance?

- The AP focuses only on fast, time-critical radio operations — so there's no delay in transmitting beacons or responding to clients.

- The WLC handles the heavy lifting like roaming, authentication, and policy centrally — meaning configurations are consistent across all APs without touching each one individually.

- Firmware updates, channel assignments, and power adjustments can be pushed from one place (the WLC) instead of logging into 50 different APs.

- Roaming between APs becomes seamless because the WLC knows the state of every client — the AP doesn't need to negotiate anything, it just hands the client off.

Q2. What is CAPWAP? Explain the Flow Between AP and Controller

CAPWAP stands for Control And Provisioning of Wireless Access Points. It's the protocol (defined in RFC 5415) that allows a lightweight AP and a WLC to communicate with each other over a network — even across routers and the internet.

Think of it like a phone line between the AP and the WLC. Everything the AP does — from learning its configuration to passing client traffic — goes through this tunnel.

The Discovery and Join Flow (step by step):

- Step 1 — AP boots up:

- The AP powers on with no configuration at all. It doesn't even have an IP address yet.

- Step 2 — DHCP:

- The AP sends a DHCP request and gets an IP address. The DHCP server may also return Option 43, which tells the AP where the WLC is.

- Step 3 — WLC Discovery:

- The AP tries to find a WLC using multiple methods: DHCP Option 43, DNS lookup for 'CISCO-CAPWAP-CONTROLLER', broadcast on local subnet, or a previously remembered WLC address.

- Step 4 — DTLS Handshake:

- Once the AP finds the WLC, they perform a DTLS (Datagram TLS) handshake to set up a secure encrypted tunnel. This prevents any rogue devices from pretending to be a controller.

- Step 5 — Join Request/Response:

- The AP sends a CAPWAP Join Request. The WLC verifies the AP's certificate and sends back a Join Response with the AP's full configuration — SSID, channel, power, VLANs, everything.

- Step 6 — Normal operation:

- The AP is now live. All management traffic flows through the CAPWAP Control tunnel, and all client data flows through the CAPWAP Data tunnel.

Q3. Where Does CAPWAP Fit in the OSI Model? The Two Tunnels and Their Purpose

OSI Layer placement:

CAPWAP runs over UDP at the Transport Layer (Layer 4), using two well-known ports:

- UDP port 5246 — Control tunnel

- UDP port 5247 — Data tunnel

Since it runs over UDP/IP, it technically spans Layer 3 (Network) and Layer 4 (Transport). The DTLS security wrapping it operates at Layer 4–5. So the short answer: CAPWAP sits at Layers 3–4.

The Two Tunnels:

| Tunnel | Port | Encrypted? | What flows through it |
| --- | --- | --- | --- |
| Control Tunnel | UDP 5246 | Yes (DTLS) | AP configuration, firmware updates, join/discovery messages, heartbeats, RF management commands |
| Data Tunnel | UDP 5247 | Optional | Actual client Wi-Fi traffic (frames from connected devices are encapsulated and sent to the WLC) |

The Control tunnel is always encrypted — you don't want someone intercepting AP configuration messages. The Data tunnel encryption is optional and often disabled in high-throughput deployments to reduce overhead.

Q4. What's the Difference Between Lightweight APs and Cloud-based APs?

| Feature | Lightweight AP (CAPWAP) | Cloud-based AP (e.g., Cisco Meraki) |
| --- | --- | --- |
| Controller location | On-premise WLC (physical hardware on your network) | Cloud (hosted by vendor — Meraki dashboard, Aruba Central, etc.) |
| Management | Through WLC CLI or GUI on local network | Through a web browser from anywhere in the world |
| Internet dependency | Not required for day-to-day operation | Required for management; APs cache config locally for operation |
| Cost model | High upfront cost (WLC hardware) | Subscription/license based (monthly or annual) |
| Scalability | Limited by WLC capacity | Easily scalable — just add APs, no hardware upgrade |
| Firmware updates | Pushed from WLC manually or scheduled | Automatic OTA updates from the cloud |
| Best for | Large enterprise with on-site IT team | Distributed offices, retail chains, schools with no local IT |
| Roaming | Fast — WLC handles everything locally | Slightly higher latency as some decisions go to cloud |

The practical difference: Lightweight APs need a WLC box sitting in your server room. Cloud-based APs just need internet access and you manage everything from a browser.

Q5. How is the CAPWAP Tunnel Maintained Between AP and Controller?

Once the AP joins the WLC and the CAPWAP tunnel is established, it doesn't just sit there passively — it's actively kept alive through a few mechanisms:

1. Heartbeat / Keepalive Messages

- The AP sends periodic Echo Request messages to the WLC through the Control tunnel.

- The WLC responds with Echo Response. This is how both sides confirm the tunnel is alive.

- Default interval is typically every 30 seconds. If the WLC misses a few consecutive heartbeats (usually 3), the AP considers the WLC unreachable.

2. DTLS Session Maintenance

- The Control tunnel runs inside DTLS (encrypted). This session has its own lifecycle and is re-negotiated if it expires or breaks.

- If the DTLS session drops, the AP re-initiates the entire join process.

3. What happens if the tunnel breaks?

- If the AP loses contact with the WLC, it enters a discovery state and tries to find another WLC (if configured with a backup WLC).

- In Local mode, the AP stops serving clients until it reconnects. In FlexConnect mode, it can continue serving clients locally using cached config.

4. AP Failover

- Enterprise deployments always configure a Primary and Secondary WLC. If the primary goes down, the AP automatically joins the secondary — this is transparent to connected clients in most cases.

Q6. What's the Difference Between Sniffer Mode and Monitor Mode?

Both modes make the AP stop serving regular clients and instead listen to the wireless environment — but for different purposes:

| Feature | Sniffer Mode | Monitor Mode |
| --- | --- | --- |
| Purpose | Capture raw 802.11 packets for analysis | Detect rogue APs, interference, and security threats |
| What it does | AP acts like Wireshark in the air — captures all frames on a channel and forwards them to an analysis tool | AP continuously scans all channels looking for unauthorized devices or interference sources |
| Output sent to | A packet capture tool (e.g., Wireshark on a laptop via RSPAN/port mirror) | The WLC — which logs findings and generates alerts |
| Client traffic? | No — AP is not serving any clients in this mode | No — AP is not serving any clients in this mode |
| Channel behavior | Fixed on one specific channel | Scans across all channels (channel hopping) |
| Use case | Troubleshooting a connectivity issue, analyzing specific frames, diagnosing interference | Security auditing, rogue AP detection, IDS/IPS enforcement, RF compliance |
| Who uses it | Network engineer investigating a specific problem | Security team doing ongoing wireless monitoring |

Easy way to remember: Sniffer mode = packet capture tool for deep-dive troubleshooting. Monitor mode = security watchdog scanning the whole environment.

Q7. If WLC is Deployed in WAN, Which AP Mode is Best for Local Network and How?

When your WLC is sitting somewhere across the WAN (like in a data center or cloud) and the APs are at a remote branch office, the best AP mode to use is FlexConnect (previously called HREAP — Hybrid Remote Edge AP).

Why FlexConnect?

In normal Local mode, every single byte of client traffic gets tunneled back to the WLC through CAPWAP before going anywhere. That means if a user at your branch office wants to load google.com, the traffic goes: User → AP → WAN → WLC (data center) → internet → back the same way. That's obviously terrible — high latency, wastes WAN bandwidth.

FlexConnect fixes this by allowing the AP to locally switch and authenticate traffic even when the WLC is far away:

- Local Switching:

- Client data is switched directly at the branch — it never goes to the WLC. The AP handles it like a traditional AP would. Local internet access is fast.

- Local Authentication:

- If the WAN link goes down, the AP can still authenticate clients using cached credentials. Clients stay connected.

- Central Management still works:

- The CAPWAP Control tunnel still goes to the WLC for management, firmware, and policy — just not the data traffic.

FlexConnect States:

| State | WLC Reachable? | What works? |
| --- | --- | --- |
| Connected | Yes | Full functionality — central or local switching depending on config |
| Standalone | No (WAN down) | AP continues serving clients using cached SSID and auth config |

For a university branch or a retail store with a remote WLC: FlexConnect is the go-to. Local traffic stays local, and the WLC stays in the loop for management only.

Q8. Challenges of Deploying Autonomous APs (50+) in a Large Network like a University

Autonomous APs are the old-school type — each one is self-contained with its own full configuration. That's fine for a small office with 3 APs, but deploying 50 or more of them in a university environment quickly becomes a serious headache:

- No Central Management:

- Every single AP needs to be configured individually — IP address, SSID, security settings, channel, power. If you want to change the Wi-Fi password, you're logging into 50+ devices one by one. That's not a workflow, that's punishment.

- No Seamless Roaming:

- Autonomous APs don't talk to each other. When a student walks from one building to another, their device drops the connection and re-associates to the new AP from scratch. In a lecture hall scenario, this is noticeable and frustrating.

- No RF Coordination:

- Each AP picks its own channel and power settings independently. This leads to co-channel interference — multiple APs on the same channel fighting each other, reducing throughput for everyone nearby.

- Security Gaps:

- There's no centralized way to detect rogue APs or unauthorized devices. Each AP only knows about its own clients. You can't do IDS/IPS at scale.

- Firmware Updates are Painful:

- To update firmware, you must access each AP individually. With 50+ APs spread across a campus, that's hours (or days) of work for every update cycle.

- Inconsistent Policies:

- Guest network, VLAN segmentation, bandwidth caps — all of these have to be configured on every AP manually. One AP misconfigured means one part of the campus behaves differently.

- No Load Balancing:

- If 40 students sit in one seminar room and all connect to the same AP, it gets overwhelmed. Autonomous APs can't redirect clients to a less-loaded nearby AP.

Bottom line: autonomous APs at scale = high admin overhead, poor roaming, inconsistent security, and RF chaos. This is exactly the problem WLCs and CAPWAP were designed to solve.

Q9. What Happens to a Wireless Client on a Lightweight AP in Local Mode if the WLC Goes Down?

This is a very important scenario to understand — and the answer depends on whether the client was already connected before the WLC went down, or is trying to connect after.

If the client is already connected when WLC goes down:

- The existing session may stay up briefly, but since the AP is in Local mode, all client data was being tunneled through CAPWAP to the WLC.

- Once the WLC is unreachable, the CAPWAP tunnel drops. The AP loses its configuration and can no longer forward traffic.

- The AP stops broadcasting the SSID (or enters a disconnected state), and the client loses Wi-Fi connectivity entirely.

If a new client tries to connect after WLC goes down:

- The AP in Local mode cannot authenticate new clients on its own — it has no local auth capability.

- The connection attempt will fail. The SSID may not even be visible anymore.

Why does this happen?

In Local mode, the AP is completely dependent on the WLC. It doesn't cache the configuration or credentials locally. It's a "dumb radio" that can't function without the brain (WLC) telling it what to do.

How to avoid this problem:

- Deploy a Secondary/Backup WLC — the AP automatically fails over if the primary goes down.

- Use FlexConnect mode instead of Local mode for remote/branch sites — in FlexConnect, the AP caches its config and can continue operating in standalone mode even without WLC connectivity.

- For critical environments (hospitals, airports), always have redundant WLC clusters.

Key takeaway: Local mode AP + no WLC = no Wi-Fi for anyone. This is a known limitation and the main reason FlexConnect exists for remote deployments.